# Manual.

How to replace the battery.

- Remove the back cover.

- Carefully peel off the double-sided tape.

- Use a screwdriver to remove the screw from battery locker.

- Take the battery off.

- Place a new battery inside the phone. There are two stickers on the other side of the battery. Don’t forget to remove the protective film from the adhesive surface on the battery.

- Put the locker back to battery connector. Do not forget to tighten the screw with a screwdriver.

- Stick the double-sided tape as shown on the picture. Do not use old double-sided tape. Use the new one!!!

- Put the back cover back on the other side of the double-sided tape.

- Congratulations, you successfully replace UMI Zero’s battery.